MICRONCONTROLLERS AND ITS APPLICATIONS LAB

## NAME: Pratyush Raj

## REG.N0: 20BEC1273

## DATE: 17-01-2022

# EXP-3 Assembly Programming with Program Control Instruction of 8051

# Lab Task 1

Aim:

To write a program to clear accumulator [A], then add 5 to the accumulator 20 times.

Software used:

Keil µvision 4 software

Program:

ORG 0000H

MOV A, #00

MOV R0, #20

LOOP: ADD A, #05

DJNZ R0, LOOP

MOV R5, A

END

Output:

Result:

We have cleared the accumulator [A] and add 5 to the accumulator 20 times which gives the final value as 64.

-------------------------------------------------------------------------------------

# Lab Task 2

Aim:

Write an 8051 ASM program to read a temperature value (T) from RAM location 55H. According to the test results, place the temperature value into the registers indicated below:

If T=30H then A=30H

If T<30H then R1=T

If T>30H then R2=T

Software used:

Keil µvision 4 software

Program:

ORG 0000H

MOV A, 55H

CJNE A, #30H, OVER

SJMP EXIT

OVER: JNC NEXT

MOV R1, A

SJMP EXIT

NEXT: MOV R2, A

EXIT: NOP

END Output:

For T>30:

For T=30:

For T<30:

Result:

We have read the value of temperature from the address D:55H and depending on the temperature value, we placed it into the different registers as mentioned in the question.

-------------------------------------------------------------------------------------

# Lab Task 3

Aim:

Write a program to move a block of FIVE data starting from RAM 40H to external ROM memory 3000H onwards and perform complement operation before storing to ROM.

Software used:

Keil µvision 4 software

Program:

ORG 0000H

MOV R0, #40H

MOV DPTR, #3000H

MOV R2, #05H

LOOP: MOV A,@R0

CPL A

MOVX @DPTR,A

INC R0

INC DPTR

DJNZ R2, LOOP

END

Output:

Result:

We moved a block of FIVE data starting from RAM 40H to external ROM memory 3000H onwards and perform complement operation before storing to ROM.

-------------------------------------------------------------------------------------

# Challenging Task 1

Aim:

To write an 8051 ASM program to add the first 15 natural numbers.

Software used:

Keil µvision 4 software

Program:

ORG 0000H

MOV R0,  #15H

MOV A,   #00H

MOV R1,  #01H

LOOP:ADD A, R1

INC R1

DJNZ R0, LOOP

MOV R4, A

END

Output:

Result:

We have calculated the sum of the 1st fifteen natural numbers and it is stored in R4 register.

-------------------------------------------------------------------------------------

# Challenging Task 2

Aim:

Write an 8051 ASM program to move a block of five data using conditional statements from 50H RAM to External ROM memory 4000H to do addition and store the result in 5000H (High Byte) and 5001H (Lower Byte).

Software used:

Keil µvision 4 software

Program:

ORG 0000H

MOV R0,#50H

MOV R1,#05H

MOV DPTR,#4000H

MOVE: MOV A,@R0

MOVX @DPTR,A

INC R0

INC DPTR

DJNZ R1,MOVE

MOV R1,#05H

MOV DPTR,#4000H

MOVX A,@DPTR

ADDR: MOV R0,A

INC DPTR

MOVX A,@DPTR

ADDC A,R0

JNC NEXT

INC R2

NEXT: DJNZ R1,ADDR

MOV A,R0

MOV DPTR,#5001H

MOVX @DPTR,A

MOV A,R2

MOV DPTR,#5000H

MOVX @DPTR,A

NOP

END

Output:

Input values in 50h lo

cation:

Printing values in

4000

H

:

Placing the sum in 5000H and 50001H:

Result:

We have transferred 5 blocks of data to external ROM 4000H and did addition and stored the result in 5000H (High Byte) and 5001H (Lower Byte).

---------------------------------------------------------------------------------------------------------------------------